| ## Main Debugging Parameters |  |  |
| --- | --- | --- |
| Number | Parameter | Function Description |
| G01.01 | Current Loop kp | Proportional adjustment of the current loop, e.g., 307/1024 = 0.3; Increasing it can accelerate the motor operation, but it will increase motor noise, reduce precision, and cause more heat generation. |
| G01.02 | Current Loop ki | Integral adjustment of the current loop, which is more subtle and precise compared to kp adjustment. It can eliminate the static error of the system. Increasing the integral can speed up the motor but will cause noise and heat generation |
| G01.03 | Current Loop kp Amplification Factor | The amplification factor for the ramp-up of the current loop proportional gain; for example, an amplification of 1.5 times equals 1024 * 1.5 = 1536. |
| G01.04 | Position Loop kp | Proportional adjustment of the position loop, e.g., 512/1024 = 0.5; Increasing it can accelerate the operation speed and reduce positioning errors, but may lead to unstable system operation. |
| G01.05 | Position Loop ki | Integral adjustment of the position loop, which is more subtle and precise compared to kp adjustment. Increasing it can eliminate steady-state position errors but will shorten the motor response time. |
| G01.06 | Speed Loop kp | Proportional adjustment of the speed loop, e.g., 26/1024 = 0.025; Increasing it can accelerate the speed tracking and reduce speed errors but may cause jitter. |
| G01.07 | Speed Loop ki | Integral adjustment of the speed loop, which is more subtle and precise compared to kp adjustment. Increasing it can eliminate steady-state speed errors but will result in a delay in speed regulation. |
| G01.08 | Speed Feedforward | Speed feedforward adjustment; increasing it can improve operation precision and response time but will cause noise and heat generation. |
| G01.09 | Acceleration Feedforward | Default is 0; increasing it can improve stability and accelerate response but will cause noise and heat generation. |

| Main Function Parameters |  |  |
| --- | --- | --- |
| Number | Parameter | Function Description |
| G02.00 | Pulse Command Filter Time | Filter time = (parameter value) * 50us; The filter time is related to the parameter and can be calculated. A larger parameter value results in deeper filtering, steeper slopes, and slower response. |
| G02.04 | Pulse Valid Edge Selection | The default valid edge should be changed according to the default edge of the specific controller. If there is a mismatch, deviations will occur during round-trip operation. |
| G02.05 | Input Pulse Frequency Limitation | Filters out high-frequency signals; increasing the value is effective when there is interference (software filtering). |
| G02.06 | Open-Loop Current Percentage | Current percentage in open-loop mode. For example: CS57: 60 corresponds to 5000mA * 60%; CS86H: 60 corresponds to 7000mA * 60%. |
| G02.07 | Closed-Loop Basic Current Percentage | Current percentage in closed-loop mode. For example: CS57: 40 corresponds to 5000mA * 40%; CS86H: 40 corresponds to 7000mA * 40%. |
| G02.08 | Closed-Loop Maximum Current Percentage | Maximum current percentage in closed-loop mode. CS57: 100 corresponds to 5000mA * 100%; CS86H: 100 corresponds to 7000mA * 100%. |
| G02.09 | Position Deviation Alarm Threshold | The range for the driver to report deviation. If the customer allows a larger deviation range or does not want the system to detect position deviation, this value can be increased. |

| Secondary Function Parameters |  |  |
| --- | --- | --- |
| Number | Name | Function Description |
| G03.00 | Alarm Output Polarity Selection | Adjustment of the driver's alarm polarity; default is transition from high level to low level; after modification, it transitions from low level to high level. |
| G03.01 | Enable Polarity Selection | Adjustment of the driver's enable polarity; default is high level active, which can be changed to low level active. |
| G03.02 | Enable Function Selection | Default is release; can be changed to input pulse blocking: no reception of input signals. |
| G03.03 | OUT Function Selection | Function adjustment of the driver's OUT terminal; default is alarm function. CS57 can be changed to in-position or brake function; CS86H can be changed to in-position function. |
| G03.04 | Out Polarity Selection | Default is output optocoupler conduction; can be changed to output optocoupler disconnection. |
| G03.05 | Positioning Completion Accuracy | The value after the driver finishes positioning and no longer adjusts. For example, 5 means 5/4 = 1.25, and no adjustment is made when the deviation is around 1 pulse. This is the accuracy value. If set too large, the accuracy will be poor. If set too small (e.g., 1), continuous adjustment may occur and not stop.。 |
| G03.06 | Encoder Resolution | Default is 4000, maximum is 20000, which can be changed as needed. |

| Speed Loop and Filter Parameters |  |  |
| --- | --- | --- |
| Number | Parameter | Function Description |
| G04.00 | Speed Segment Node 1 (Low Speed) | Unit: r/s. The internal PI parameters are segmented, and the PI parameters for the three segments (low speed, medium speed, and high speed) increase in proportion to the segments. The segment nodes can be adjusted according to actual applications. |
| G04.01 | Speed Segment Node 2 (High Speed) |  |
| G04.02 | Given Speed First-Order Filter | Given speed first-order filter coefficient. |
| G04.03 | Given Speed Second-Order Filter | Given speed second-order filter coefficient. |
| G04.04 | Feedback Speed First-Order Filter | Feedback speed first-order filter coefficient. |
| G04.05 | Feedback Speed Second-Order Filter | Feedback speed second-order filter coefficient. |
| G04.06 | Acceleration Filter | Acceleration filter coefficient, mainly used to improve vibration caused by repeated rapid start-stop with large acceleration changes. |
| G04.07 | Position Loop Output Filter | Position loop output filter coefficient, which can reduce sudden changes caused by external disturbances. |
| G04.08 | Speed Loop Output Filter | Speed loop output filter coefficient, which can reduce sudden changes caused by external disturbances. |
| G04.09 | Position Loop kp2 | Position loop proportional gain in high-speed segment. |

| Spontaneous Pulse Parameters |  |  |
| --- | --- | --- |
| Number | Parameter | Function Description |
| G07.00 | DIP Switch Corresponding Speed Point 0 | SW5/6/7/8 ON/ON/ON/ON |
| G07.01 | DIP Switch Corresponding Speed Point 1 | SW5/6/7/8 OFF/ON/ON/ON |
| G07.02 | DIP Switch Corresponding Speed Point 2 | SW5/6/7/8 ON/OFF/ON/ON |
| G07.03 | DIP Switch Corresponding Speed Point 3 | SW5/6/7/8 OFF/OFF/ON/ON |
| G07.04 | DIP Switch Corresponding Speed Point 4 | SW5/6/7/8 ON/ON/OFF/ON |
| G07.05 | DIP Switch Corresponding Speed Point 5 | SW5/6/7/8 OFF/ON/OFF/ON |
| G07.06 | DIP Switch Corresponding Speed Point 6 | SW5/6/7/8 ON/OFF/OFF/ON |
| G07.07 | DIP Switch Corresponding Speed Point 7 | SW5/6/7/8 OFF/OFF/OFF/ON |
| G07.08 | DIP Switch Corresponding Speed Point 8 | SW5/6/7/8 ON/ON/ON/OFF |
| G07.09 | DIP Switch Corresponding Speed Point 9 | SW5/6/7/8 OFF/ON/ON/OFF |
| G07.10 | DIP Switch Corresponding Speed Point 10 | SW5/6/7/8 ON/OFF/ON/OFF |
| G07.11 | DIP Switch Corresponding Speed Point 11 | SW5/6/7/8 OFF/OFF/ON/OFF |
| G07.12 | DIP Switch Corresponding Speed Point 12 | SW5/6/7/8 ON/ON/OFF/OFF |
| G07.13 | DIP Switch Corresponding Speed Point 13 | SW5/6/7/8 OFF/ON/OFF/OFF |
| G07.14 | DIP Switch Corresponding Speed Point 14 | SW5/6/7/8 ON/OFF/OFF/OFF |
| G07.15 | DIP Switch Corresponding Speed Point 15 | SW5/6/7/8 OFF/OFF/OFF/OFF |

Software Parameter Saving Method：

After modifying the parameters, first click the download button. After the operation is completed, click "Save Power-Off Parameters". After the success prompt, power off and restart the driver, and the parameters will be saved successfully.